SOC Lab — Firewall Rule Documentation

pfSense (Netgate 2100)  |  Router-on-a-stick  |  Default-deny policy

DESIGN PRINCIPLES

All inter-VLAN traffic is denied by default; only the rules below permit traffic. Each VLAN is segmented so that the Attack and Victim networks cannot reach the Management or SIEM planes. Isolated VLANs (Victim, Attack) have no internet access, enforced implicitly by the absence of any WAN-permit rule under the default-deny policy.

INTERFACE / VLAN ASSIGNMENTS

| Interface | VLAN | Network | Role |
| --- | --- | --- | --- |
| LAN / parent | — | mvneta1 | VLAN trunk (router-on-a-stick) |
| 10MGMT | VLAN 10 | 10.0.10.0/24 | Management |
| 20SIEM | VLAN 20 | 10.0.20.0/24 | SIEM Monitoring |
| 30VICTIM | VLAN 30 | 10.0.30.0/24 | Victim |
| 40ATTACK | VLAN 40 | 10.0.40.0/24 | Attack |

FIREWALL RULES BY INTERFACE

WAN

| Action | Source | Destination | Port | Description |
| --- | --- | --- | --- | --- |
| Block | RFC 1918 networks | * | * | Block private networks |
| Block | Reserved / not assigned by IANA | * | * | Block bogon networks |

LAN

| Action | Source | Destination | Port | Description |
| --- | --- | --- | --- | --- |
| Pass | (this firewall) | * | * | Anti-Lockout Rule |
| Pass | LAN subnets | * | * | Default allow LAN to any |
| Pass | LAN subnets | 10.0.10.5 | 443 / 80 | Allow LAN to Proxmox on VLAN 10 |

10MGMT — Management

| Action | Source | Destination | Port | Description |
| --- | --- | --- | --- | --- |
| Pass | 10MGMT subnets | * | * | Allow mgmt to all |

20SIEM — SIEM Monitoring

| Action | Source | Destination | Port | Description |
| --- | --- | --- | --- | --- |
| Pass | 20SIEM subnets | 30VICTIM subnets | * | Allow Nessus scan to victims |
| Pass | 20SIEM subnets | 10MGMT subnets | * | SIEM to Mgmt |

30VICTIM — Victim

| Action | Source | Destination | Port | Description |
| --- | --- | --- | --- | --- |
| Pass | 30VICTIM subnets | * | * | Allow victim gateway |
| Pass | 30VICTIM subnets | 10.0.20.20 | 1514–1515 | Wazuh agent to manager |
| Pass | 30VICTIM subnets | 40ATTACK subnets | * | Allow victim to attack |
| Block | 30VICTIM subnets | 10MGMT subnets | * | Block victim to mgmt |
| Block | 30VICTIM subnets | 20SIEM subnets | * | Block victim to SIEM |

40ATTACK — Attack

| Action | Source | Destination | Port | Description |
| --- | --- | --- | --- | --- |
| Pass | 40ATTACK subnets | 40ATTACK address | * | Allow attack to gateway |
| Pass | 40ATTACK subnets | 30VICTIM subnets | * | Attack to Victim (Logged) |
| Block | 40ATTACK subnets | 10MGMT subnets | * | Block attack to mgmt |
| Block | 40ATTACK subnets | 20SIEM subnets | * | Block attack to SIEM |

SEGMENTATION SUMMARY

| Control | Enforcement |
| --- | --- |
| Attack → Management | Blocked |
| Attack → SIEM | Blocked |
| Victim → Management | Blocked |
| Victim → SIEM | Blocked (except Wazuh agent telemetry, ports 1514–1515) |
| Victim / Attack → Internet | Blocked (default-deny, no WAN permit rule) |
| Attack → Victim | Permitted and logged (attack simulation path) |