Network & Hardware Documentation

Upper Peninsula Cybersecurity Institute  |  Domain: siemlab.local

NETWORK LAYOUT (VLANS & IP ASSIGNMENTS)

| VLAN | Purpose | Subnet | Gateway | Host | IP Address |
| --- | --- | --- | --- | --- | --- |
| 10 | Management | 10.0.10.0/24 | 10.0.10.1 | Proxmox Host | 10.0.10.5 |
| 10 | Management | 10.0.10.0/24 | 10.0.10.1 | Acer (Mgmt Laptop) | 10.0.10.10 |
| 20 | SIEM | 10.0.20.0/24 | 10.0.20.1 | Security Onion | 10.0.20.10 |
| 20 | SIEM | 10.0.20.0/24 | 10.0.20.1 | Wazuh Manager | 10.0.20.20 |
| 30 | Victim | 10.0.30.0/24 | 10.0.30.1 | DC01 (Active Directory) | 10.0.30.10 |
| 30 | Victim | 10.0.30.0/24 | 10.0.30.1 | Metasploitable 3 | 10.0.30.30 |
| 30 | Victim | 10.0.30.0/24 | 10.0.30.1 | Windows 11 Victim | 10.0.30.40 |
| 30 | Victim | 10.0.30.0/24 | 10.0.30.1 | DVWA | 10.0.30.80 |
| 40 | Attack | 10.0.40.0/24 | 10.0.40.1 | Kali Linux | 10.0.40.200 |
| 30 / 40 | VM Host | — | — | Legion (bridges victim & attack VMs) | 10.0.30.100 / 10.0.40.102 |
| 1 (Native) | Default / Mgmt | 192.168.1.0/24 | — | pfSense (Netgate 2100) | 192.168.1.1 |

Note: Kali Linux is documented at its assigned static address (10.0.40.200); it currently pulls a DHCP lease (10.0.40.101) and is pending static assignment.

HARDWARE INVENTORY

| Device | Role | Key Specifications |
| --- | --- | --- |
| Custom Desktop (Proxmox Host) | SOC server — hosts SIEM VMs | Ryzen 5 7600X, 32GB DDR5, RTX 5070, 2TB WD Black SN850X SSD |
| Lenovo Legion 7i Gen 10 | Hosts attacker & victim VMs (VirtualBox) | Core Ultra 7 255HX (20 cores), 32GB RAM, RTX 5060, 1TB SSD |
| Acer Nitro AN515-43 | Management workstation | Ryzen 5 3550H, 16GB RAM |
| Netgate 2100 | Router / firewall (pfSense) | Routing, DHCP, default-deny firewall |
| UniFi USW-Pro-Max-16-PoE | Layer-2 switch | VLAN trunking, SPAN/port mirroring |
| CyberPower UPS | Battery backup | Power protection for lab |

PROXMOX VIRTUAL MACHINE INVENTORY

| VM ID | Name | RAM | Boot Disk | Status |
| --- | --- | --- | --- | --- |
| 101 | Security-Onion | 16 GB | 400 GB | Running |
| 102 | wazuh-manager | 4 GB | 60 GB | Running |

Proxmox VE 9.2.2. SIEM platforms run as VMs on the Proxmox host; attacker and victim systems run on the Legion via VirtualBox.

SWITCH PORT & CABLE MAP

| Port | Cable Color | Connected To |
| --- | --- | --- |
| 1 | Yellow | Netgate 2100 (pfSense uplink) |
| 2 | Blue | Proxmox — Management |
| 3 | Black | SPAN mirror (Proxmox sensor input) |
| 4 | Red | Victims (Legion) |
| 5 | Orange | Acer |
| 6 | Purple | Proxmox — Wazuh |
| 7 | Green | Kali (Legion) |
| 8 | Pink | Management laptop — UniFi/pfSense GUI access (intermittent; currently unplugged) |

PROXMOX NETWORK BRIDGES

| Bridge | Physical Port | Purpose |
| --- | --- | --- |
| vmbr1 | Management NIC | Proxmox management (10.0.10.5) |
| vmbr2 | enp6s0f0 | SPAN input from mirror port |
| vmbr3 | enp6s0f1 | Dedicated Security Onion management |