Interface & Bridge Mapping

UPCI SOC/SIEM Independent Study  •  Tristen Dennis  •  Supervisor: Michael Sauer

This document maps every network interface and bridge across the lab environment: the Proxmox host bridges, each virtual machine's network devices, and the physical adapters on the Legion and Acer. It shows how each system connects to its VLAN and how mirrored traffic reaches the Security Onion sensor.

PROXMOX HOST BRIDGES

The Proxmox host carries all bridges. Each bridge maps to a physical NIC:

| Bridge | Physical Port | IP / Purpose |
| --- | --- | --- |
| vmbr0 | enx9c69d3218074 (USB-C NIC) | Manual (unused / spare) |
| vmbr1 | onboard NIC | 10.0.10.5/24, gw 10.0.10.1 — Management |
| vmbr2 | enp6s0f0 | SPAN input from mirror port |
| vmbr3 | enp6s0f1 | Dedicated SIEM / SO management |

PROXMOX VIRTUAL MACHINES

The three SIEM platforms run on Proxmox. Their network devices map to bridges as follows:

| VM | Device | Bridge | Internal NIC | Role |
| --- | --- | --- | --- | --- |
| 101 — Security Onion | net0 | vmbr3 | ens18 | Management (10.0.20.10) |
| 101 — Security Onion | net1 | vmbr2 | bond0 | Monitoring (SPAN) |
| 101 — Security Onion | net2 | vmbr2 | bond0 | Monitoring (SPAN) |
| 102 — Wazuh | net0 | vmbr3 | ens18 | SIEM (10.0.20.20) |
| 103 — Nessus | net0 | vmbr3 | ens18 | Scanner (10.0.20.30) |

Security Onion bonds its two monitoring interfaces (net1 and net2, both on vmbr2) into a single interface, bond0, which receives mirrored traffic from the SPAN path. Its management interface (net0) is kept separate on vmbr3.

LEGION — PHYSICAL ADAPTERS

The Legion hosts the attacker and victim VMs in VirtualBox. It uses two USB Ethernet adapters, one per VLAN, plus VirtualBox's internal host-only adapters (not part of the lab network).

| Adapter | VLAN | Legion IP |
| --- | --- | --- |
| Realtek USB GbE Family Controller | VLAN 30 (Victim) | 10.0.30.100 |
| Realtek Gaming USB 2.5GbE Family Controller | VLAN 40 (Attack) | 10.0.40.102 |
| VirtualBox Host-Only Adapter(s) | Internal (VirtualBox) | Not part of lab VLANs |

LEGION — VIRTUAL MACHINES (VIRTUALBOX)

Each VirtualBox VM is bridged to one of the Legion's two USB adapters, placing it on the correct VLAN:

| VM | Bridged Adapter | Internal NIC | VLAN | IP |
| --- | --- | --- | --- | --- |
| DC01 (Active Directory) | Realtek USB GbE | Ethernet | VLAN 30 | 10.0.30.10 |
| Metasploitable 3 | Realtek USB GbE | eth0 | VLAN 30 | 10.0.30.30 |
| Windows 11 Victim | Realtek USB GbE | Ethernet * | VLAN 30 | 10.0.30.40 |
| DVWA | Realtek USB GbE | enp0s3 | VLAN 30 | 10.0.30.80 |
| Kali Linux | Realtek Gaming USB 2.5GbE | eth0 | VLAN 40 | 10.0.40.200 |

* The Windows 11 victim was not powered on at the time of documentation; its interface name is expected to be “Ethernet”, matching the other Windows host (DC01).

ACER (MANAGEMENT WORKSTATION)

| System | VLAN | IP |
| --- | --- | --- |
| Acer Nitro | VLAN 10 (Management) | 10.0.10.10 |

TRAFFIC & MONITORING PATH

Mirrored (SPAN) traffic from the switch enters the Proxmox host on enp6s0f0 (vmbr2) and is forwarded to Security Onion's bonded monitoring interface (bond0). The SPAN bridge has MAC learning disabled and flooding enabled so a full copy of the traffic reaches the sensor; this is applied automatically on each boot by a startup script. Management traffic is kept on a separate path (vmbr3 for the SIEM VMs, vmbr1 for the host).